Component Heritage Summary

Component Contents:

- Prior Flight Heritage

- Environmental History

- Known Limitations

- Radiation and Thermal Constraints

1    ZES Board

- Prior Flight Heritage

  - Flight legacy in KYUTECH (Kyushu Inst. of Tech.-Japan) launched in Oct./'25 for a LEO mission. NASA-AMES has validated the board for launch, expected in mid/'26 for the EdTech mission.

- Environmental History

  - Simulation data mentions a 5.88-year mission lifetime. Technical explanation supports sustaining 2 years in LEO at 500KM.

- Known Limitations

  - Mission lifetime is limited by radiation, though it will be extended as the board will have further shielding on the overall system.

- Radiation and Thermal Constraints

  - No specific constraints.

2    Rila Dosimeter

- Flight Heritage

  - Heritage with the Liulin missions. Similar instruments used on the ISS, Chandrayaan-1, and Ten-Koh Satellite.

- Environmental History

  - Has not been flown before.

- Known Limitations

  - Volume is 168 cm³.

  - Dimensions are 80x70x30 mm.

  - Requires 12 VDC.

  - Power consumption is 0 mA (Sleep), 0.35 W (Working), and < 0.5 W (Peak).

  - Limited to LEO (500 km altitude).

  - Mission lifetime is not less than 2 years with a potential of 5 years.

- Radiation and Thermal Constraints

  - Nominal thermal window is -20°C to +40°C.

  - Survival window is -40°C to +70°C (Device must be OFF).

3    Primary Accelerometer: MS1002-class

- Flight Heritage

  - Used in aerospace and defense for inertial navigation, tilt sensing,and platform stabilization. Considered a high-reliability industrial component.

- Environmental History

  - Considered a high-reliability industrial component.

- Known Limitations

  - Operates at a regulated 3.3V DC power rail.

  - Power consumption is 10 mW.

  - Mass is 1.5 grams.

  - Analog voltage output type.

  - Can withstand vibration up to 6000 g.

- Radiation and Thermal Constraints

  - Operating temperature range of -40 °C to +125 °C.

  - Storage range of -55 °C to +125 °C.

4    Secondary Accelerometer: ADXL355-class MEMS

- Flight Heritage

  - Frequently utilized in CubeSat ADCS and vibration monitoring on launch vehicles.

- Environmental History

  - Operating Temp: -40C to +125C

  - Can survive high-G launch environments.

- Known Limitations

  - 3-Axis digital output (SPI/I2C).

  - Mass is 0.26 grams.

  - Operates at 2.25V to 3.6V DC.

- Radiation and Thermal Constraints

  - No specific constraints beyond operating temperature.

5    Magnetometer

- Flight Heritage

  - None

- Environmental History

  - None

- Known Limitations

  - Operates with a +3.3V (+/-TBDV) spacecraft power supply.

  - Sampling power consumption ≤ 65 mW + TBD RS-422 overhead.

  - Idle power consumption ≤ 15 mW.

- Radiation and Thermal Constraints

  - Must perform nominal operations within an operating temperature range of -20°C to +45°C.