ICD-101: Bus to Payload Data Interface

1. Document Control

- Document Number: SDSU-SB+-ICD-101

- Revision: v1.0

- Date: March 2026

- Owner: SDSU Payload Team

2. Purpose

This document defines the data and command interface between the SDSU payload subsystem (via ZES) and the spacecraft OBC.

3. Scope

This ICD governs:

- Data transmission between ZES and OBC

- Telemetry structure and formatting

- Command-response behavior

- Data integrity and robustness

4. Interface Overview

Data Flow:
RILA → ZES → OBC → Ground

ZES acts as:

- Data aggregator

- Packet formatter

- Communication interface

5. Electrical Interface

- Protocol: UART

- Logic level: 3.3 V

- Baud rate: 115200 bps

- Communication: Full-duplex

6. Data Structure

6.1 Frame Format

- Sync bytes (2B)

- Header (type, length)

- Payload

- CRC-16 checksum

6.2 Data Modes

| Mode | Description | Size | Baud Rate | Connection |
| --- | --- | --- | --- | --- |
| Mode A | Full spectrum data | ~522 bytes | 115,200bps | UART (3.3 V) |
| Mode B | Summary packet | ≤340 bytes | 115,200bps | UART (3.3 V) |

- Data Packets Include a 1 Byte head and tail.

- Main Difference Between Modes is Packet Size

7. Command & Response

- OBC polls ZES for data

- ZES responds with buffered frames

- ZES manages data queue and prioritization

8. Timing

- OBC response time: ≤100 ms

- Frame transmission: ~45 ms

- Data collection interval: configurable (30–600 s)

9. Telemetry Robustness & Error Handling

Corrupted Frames

- Detected via CRC-16

- Discarded

- Retransmission requested (≤3 attempts)

Dropped Frames

- Detected via sequence number or timing gaps

- Logged in telemetry

Buffering

- Minimum 4 frames stored

- Store-and-forward behavior

10. Verification

Tests

- UART communication test

- CRC validation test

- Frame parsing test

- One-hour continuous operation test

- Data integrity verification

Acceptance Criteria

- ≥99.9% valid frames

- No unexpected resets

- Correct packet decoding