ICD-106: Payload EMI/EMC Interface

1. Document Control

- Document Number: SDSU-SB+-ICD-106

- Revision: v1.0

- Date: March 2026

- Owner: SDSU Payload Team

2. Purpose

This document defines the EMI/EMC interface of SDSU payload subsystems, including RILA, ZES, and supporting sensors. It defines electrical noise, grounding, shielding, and susceptibility assumptions.

3. Scope

This ICD governs:

- Establish electrical noise assumptions

- Grounding assumptions

- Shielding assumptions

- Susceptibility assumptions

4. Electrical Noise/ EMI/EMC

- MS1002 Accelerometer:

  - Accelerometers will be provided in an antistatic bag with ESD warning labels on it.

  - Only remove the accelerometer from the packaging when it is being used.

  - Manipulate the device within an ESD-controlled environment.

  - Store devices in a shielded environment that protects against ESD damage.

  - Wear a wrist strap and use ESD-safe gloves when handling the device.

  - The MS1002 analog output is sensitive to external electromagnetic interference, particularly high-frequency digital switching noise and parasitic capacitance. Noise coupling into this interface shall be minimized through grounding, shielding, and filtering prior to ADC conversion.

- Rila:

  - RILA communicates with the ZES using an RS-422 differential serial interface.

  - Differential signaling provides inherent immunity to common-mode electromagnetic noise and reduces radiated emissions.

  - Expected emissions from the RILA interface are limited due to controlled impedance differential signaling. Susceptibility is minimized through shielded twisted-pair cabling and proper termination.

  - Data integrity is maintained through CRC validation and bounded retransmission (≤3 attempts), ensuring resilience to transient EMI-induced errors.

- ZES:

  - ZES is the primary digital processing unit and is the dominant internal source of electrical noise.

  - Expected emissions are generated from microcontroller switching activity, serial communication interfaces (UART, SPI, I²C), and DC/DC power conversion.

  - ZES is designed for high-reliability environments and includes single-event latch-up (SEL) protection, single-event upset (SEU) mitigation, and error detection and correction (EDAC), which reduce susceptibility to transient disturbances and data corruption.

  - All interfaces operate at 3.3 V logic levels and share a common ground reference to limit noise coupling.

- Magnetometer:

  - Will need to be distanced from electronic components like motors, high‑current lines, reaction wheels, batteries, and deployable solar panels to improve measurement accuracy.

  - The magnetometer is susceptible to both electromagnetic interference and magnetic field disturbances. EMI mitigation is achieved through physical separation from noise sources rather than shielding.

- System-Level EMI/EMC Requirement:

  - The payload EMI/EMC behavior is bounded such that all communication interfaces maintain required data integrity under expected spacecraft electromagnetic conditions, as verified through CRC validation, retransmission limits, and system-level communication testing.

5. Grounding/Shielding/Susceptibility

- MS1002 Accelerometer:

  - The sensor is sensitive to external parasitic capacitance, thus a ground plane directly below the accelerometer is recommended for shielding purposes.

  - Additional filtering and short analog signal paths shall be used to reduce susceptibility to high-frequency noise coupling from digital subsystems.

- Rila:

  - RS-422 cabling shall use twisted shielded pairs (100–120 Ω).

  - Shield termination only at ZES end to prevent ground loops.

  - Cabling shall be routed to minimize loop area and avoid proximity to sensitive analog and magnetic components. Differential signaling reduces susceptibility to external EMI.

  - Cabling shall be routed separately from power lines where feasible to reduce coupling.

- ZES:

  - 2mm shielding of aluminum is implemented for radiation (TID) protection and structural considerations. EMI mitigation at the ZES level is primarily achieved through grounding, PCB layout, and controlled signal routing.

  - ZES shall serve as the central grounding reference for the payload, with all subsystem grounds tied to a common node. A star-grounding approach shall be used where possible to minimize ground loops.

  - PCB-level grounding and controlled routing shall be used to limit emission and coupling between digital and analog subsystems.

- Magnetometer:

  - No shielding is required.

  - The magnetometer shall rely on spatial separation from electromagnetic interference sources as the primary mitigation method. Placement shall avoid proximity to high-current conductors and switching electronics.