ICD-100: Bus to Payload Power Interface

1. Document Control

- Document Number: SDSU-SB+-ICD-100

- Revision: v1.0

- Date: March 2026

- Owner: SDSU Payload Team

2. Purpose

This document defines the electrical power interface between the spacecraft bus (EPS) and the SDSU payload subsystem, including RILA, ZES, and supporting sensors. It establishes voltage levels, current limits, distribution architecture, and verification requirements.

3. Scope

This ICD governs:

- Bus power delivery to payload

- Internal payload power distribution via ZES

- Power sequencing and control behavior

- Grounding and protection requirements

4. Power Architecture Overview

Primary Power Flow:

- EPS → 5 V → ZES (primary controller)

- EPS → 12 V → RILA (science instrument)

Secondary Distribution:

- ZES → 3.3 V → Magnetometer, Accelerometers, RS-422 converter

5. Electrical Requirements

5.1 Power Requirements Table

| Component | Voltage | Nominal Current | Peak Current | Notes |
| --- | --- | --- | --- | --- |
| RILA | 12 V ±0.5 V | ~30 mA | ~42 mA | Direct EPS supply |
| ZES | 5 V ±5% | 120–200 mA | ≤500 mA | Main controller |
| Magnetometer | 3.3 V | ≤15 mW | ≤65 mW | via ZES |
| ADXL355 | 2.25–3.6 V | low | TBD | via ZES |
| MS1002 | 3.2–3.4 V | 2.3 mA | 4 mA | via ZES |

6. Power Sequencing

- ZES shall power on first

- ZES logic must stabilize before sensor enable

- RILA power may be controlled via ON/OFF signal or bus command

- Safe-mode condition → payload power disabled

7. Grounding & Bonding

- All payload elements share a common ground

- Ground reference established at ZES input

- RS-422 cable shields terminated at ZES side only

- Star grounding topology recommended

8. Protection & Limits

- ZES input voltage shall not exceed 5.25 V

- UART logic shall not exceed 3.6 V

- RILA input shall not exceed 13 V

- RS-422 converter shall include ESD protection

- ADC/sensor inputs protected against overvoltage

9. Verification

Tests

- Power rail validation (5 V / 12 V)

- Current draw measurement

- Power sequencing validation

- Safe-mode shutdown test

- Thermal behavior under load

Acceptance Criteria

- Voltage within tolerance

- Current within expected range

- No unexpected resets or overcurrent