### 1. Document Control

- Document Title: SDSU-SentinelBio+-Payload-Mounting-Configuration

- Revision: A

- Date: 29 March 2026

- Prepared By: Jayden

- Approved By:

- Configuration Management ID (if applicable):

### 2. Purpose

The purpose of this Mechanical Interface Document is to describe the physical parameters of the mounting layout for each component of the payload subsystem. These components include the RILA Dosimeter, the ZES ZSOM-M01 processing board, the MS1002 Accelerometer, the ADXL355-class MEMS accelerometer, a Printed circuit board (PCB), and a not yet fully defined Magnetometer. This document will provide measurements for where each component will be secured to the satellite bus, and can be referenced for future assembly of components.

### 3. Scope

This document describes specific mounting details for each component of the payload, including descriptions of mounting configuration, exact measurements from the edges of the satellite bus side plates to the location of screws and connectors. This document will detail mounting method specifications such as screws, spacers, mounting brackets, thread types, hole diameters, and screw configurations. Relevant diagrams will be included as necessary.

### 4. Referenced Documents

| Document Title | Document Number | Revision |  |  |
| --- | --- | --- | --- | --- |
| SDSU-SentinelBio+-ICD-ADXL355-ZES | DOC-001 | Rev B | 2025-11-24 |  |
| ZSOM-M01 Datasheet v1.6a | DOC-002 | Rev A | 2024-12 |  |
| SDSU-SentinelBio+-ICD-RILA-ZES | DOC-003 | Rev B | 2025-11-23 |  |
| SDSU-SentinelBio+-ICD-MS1002-ZES | DOC-004 | Rev A | 2025-12-9 |  |
| MEMS digital output motion sensor: ultra-low-power high-performance 3-axis "nano" accelerometer SDSI Technical Memorandum SB+ Mission Accelerometer Selection and Allocation Quad-Mag ICD | DOC-005 DOC-006 DOC-007 | Rev A Rev A Rev A | 2016-10 2025-24-11 Undated |  |

### 5. Mechanical Integration Overview

#### 5.1 ZSOM-M01 ZES Processing Board

- Component Connections

  - Wired connection to RILA, both Accelerometers through PCB, Magnetometer, and onboard computer (OBC) (Refer to  DOC-001, DOC-003, DOC-004, and DOC-007 for connection types)

- Mounting description

  - ZES board sits on larger board that gives pin connections to other components

  - This board is screwed onto threaded brass spacer legs that are screwed into one of the inner walls of the chassis

#### 5.2 RILA Dosimeter

- Connections

  - Wired connection to ZES Board (Refer to DOC-003 for connection types)

- Housing

  - Box shell integrated with component

- Mounting description

  - Tabs stick out of two sides of box

  - 2 holes in each tab, 4 holes total, unthreaded

  - Mounted with nut, bolt, and washers in each hole, bolted into the wall of the chassis, opposite wall to the ZES board

#### 5.3 Magnetometer (not fully defined)

- Connections

  - Wired connection to ZES Board (Refer to DOC-007 for some information regarding connection types)

- Housing

  - Not fully defined

- Mounting description (current assumptions)

  - Bolted on thin square mounting bracket that is bolted onto chassis inner wall

  - The bracket is a thin rectangular metal plate with perpendicular tabs on two of its edges, extending on both sides of the plate.

  - This mounting bracket is positioned in the midplane of the chassis, parallel with and situated between the 2 walls that the ZES board and the RILA box are secured to

#### 5.4 MS1002 Accelerometer

- Connections

  - Wired connection to ZES Board through PCB (Refer to DOC-004 for connection types)

- Mounting description

  - Soldered directly onto PCB alongside ADXL355 Accelerometer

#### 5.5 ADXL355 Accelerometer

- Connections

  - Wired connection to ZES Board through PCB (Refer to DOC-001 for connection types)

- Mounting description

  - Soldered directly onto PCB alongside MS1002 Accelerometer

#### 5.6 Printed Circuit Board (PCB)

- Connections

  - Wired connection to ZES Board (Refer to DOC-003 for connection types)

- Mounting description

  - Secured to the same mounting bracket as the magnetometer, but on the opposite side

### 6. Mounting Hardware Specifications

#### 6.1 midplane bracket

- Part Size

  - Width: 2 mm (bracket thickness)

    - Total width including tabs: 32mm

  - Length of 96.16 mm (entire span between opposite walls)

  - Height: 110 mm

  - Tabs on each side flush with chassis inner wall, extending 15 mm past bracket thickness

- Hole size and location

  - 8 holes, 4 on each side, 2 on each tab

  - Holes are 4 mm diameter

  - Center point of each hole is located 7.5 mm away from tab edge, 7.5 mm away from wall

#### 6.2  Screws and Bolts

- ZES board screws

  - M2 screws, 8 total, 4 connecting spacers to ZES board, 4 connecting spacers to chassis wall

- RILA box bolt assembly

  - 4 M4 nuts, 4 M4 hex head bolts

- Midplane Bracket bolt assembly

  - 8 M4 nuts, 8 M4 hex head bolts

- Magnetometer bolt assembly (current assumption)

  - 4 M4 nuts, 4 M4 hex head bolts (subject to change)

- PCB bolt assembly

  - 4 M3 nuts, 4 M3 hex head bolts

#### 6.3 Spacer Legs

- Part Size

  - 10 mm lengthwise

- Hole size and location

  - M1 threads through both ends

#### 5.4 PCB

- Part Size

  - The PCB will be cut as a 95x95 mm square to fit width-wise against inner wall of chassis

- Holes

  - 3 mm diameter holes in each corner of board

  - The hole centerpoints are located 8 mm away from PCB edge top to bottom, and 7 mm away from PCB edge side to side

### 7. Mounting Location Parameters

#### 7.1 ZSOM-M01 ZES Processing Board and Spacers

- ZES board measures 90.17 x 96.89 mm and will be situated with the 96.89 dimension running top to bottom against the inner wall

- Distance from bottom end of chassis to spacer holes

  - 145 mm

- Each hole is an equal distance from the edge of its respective wall

#### 7.2 RILA Dosimeter

- Distance from bottom end of chassis to RILA box holes

  - 146 mm

- Each hole is an equal distance from the edge of its respective wall

#### 7.3 Magnetometer (not fully defined)

- Distance from bottom end of chassis to magnetometer holes through midplane spacer

  - Exact size unknown, will update

- Each hole is an equal distance from the edge of the midplane bracket side to side, but no specific known value

#### 7.4 MS1002 Accelerometer

- Soldering location

  - Top edge 15 mm away from top edge of PCB

  - Left edge 20 mm away from left edge of PCB

#### 7.5 ADXL355 Accelerometer

- Soldering location

  - Bottom edge 15 mm away from bottom edge of PCB

  - Left edge 20 mm away from left edge of PCB

#### 7.6 Printed Circuit Board (PCB)

- Bottom hole midpoint distance from bottom end of chassis

  - 145 mm

- Bottom hole center point distance from bottom of midplane bracket

  - 10 mm

### 11. Signature Block and Revision Control

| Team | Name | Signature | Date |
| --- | --- | --- | --- |
| SDSC | [Insert Name] | __________________ | 2025-MM-DD |
| SDSI | [Insert Name] | __________________ | 2025-MM-DD |
| SDSU | [Insert Name] | __________________ | 2025-MM-DD |

| Rev. | Date | Description | Editor |
| --- | --- | --- | --- |
| A | 2026-3-29 | Initial Draft | Jayden Thomas |