ICD-105 v2: Payload Thermal Interface

1. Document Control

- Document Number: SDSU-SB+-ICD-105

- Revision: v1.0

- Date: March 2026

- Owner: SDSU Payload Team

2. Purpose

This document defines the thermal interface of SDSU payload subsystems, including RILA, ZES, and supporting sensors. It establishes operating and storage temperatures, and thermal constraints for soldiering. This ICD will also examine the expected internal heat dissipation, radiative assumptions, and any thermal isolation or coupling requirements.

3. Scope

This ICD governs:

- Operating temperatures of payload subsystems

- Storage temperatures of payload subsystems

- Soldiering constraints if applicable

- Internal heat dissipation

- Radiative and isolation/coupling assumptions

4. Operating and Storage Temperatures

- ADXL355 Accelerometer:

  - Operating: -40℃ to +125℃

  - Storage: -55℃ to +150℃

- MS1002 Accelerometer:

  - Operating: -40℃ to +125℃

  - Storage: -55℃ to +125℃

- Rila:

  - Operating:  -20℃ to +40℃

- Magnetometer:

  - Operating: -20℃ to +45℃

- ZES:

  - Operating: -40℃ to +105℃

4.1 Thermal Constraints

Table 4.1. Soldering Thermal Requirements for MS1002 Accelerometer

| Phase | Sn/Pb |  | Pb-Free |  |
| --- | --- | --- | --- | --- |
|  | Duration [sec] | Temperature [°C] | Duration [sec] | Temperature [°C] |
| Peak | 10-30 | 235-240 | 20-40 | 245-250 |
| Fusion | 60-150 | 183 | 60-150 | 217 |
| Preheat | 60-120 | Min: 100 Max:150 | 60-180 | Min: 150 Max: 200 |

5. Internal Heat Dissipation

- ADXL355 Accelerometer:

  - Peak Operating Estimate: 0.036 Watts or ~ 1 °C (Testing Required to verify)

- MS1002 Accelerometer:

  - Peak Operating Estimate: 0.136 Watts or ~ 3 °C (Testing Required to verify)

- Rila:

  - Peak Operating Estimate: 0.504 Watts or ~ 11.5 °C (Testing Required to verify)

- Magnetometer:

  - Peak Operating Estimate: 0.2145 Watts or ~ 5 °C (Testing Required to verify)

- ZES:

  - Operating: +21.61℃ to +25.11℃, Peak 25.72℃

6. Worst Case Scenarios

- The average temperature range for a 500km LEO orbit is -100°C to 120°C.

  - So, for the worst-case hot scenario, the CubeSat will need to account for the average 120 °C temperature as well as ensure proper radiative approach to account for the internal heat dissipation of the payload.

  - For the worst-case cold scenario, the CubeSat will need to account for the average -100°C temperature. This can be mitigated through proper insulation and thermal coupling of key payload components and their internal heat dissipation.

7. Radiative Assumptions

- ADXL355 Accelerometer:

- MS1002 Accelerometer:

- Rila: Awaiting vendor specific information regarding placement of the sensor based on        	radiation collection.

- Magnetometer:

- ZES:

8. Isolation/Coupling Requirements

- Magnetometer does have a keep out zone that is being established by the vendor; thus, it will require thermal isolation. This will be added once information is available.

- Rila shall also be thermal coupled with the CubeSat along with the other payload components, as it has a lower temperature range.

  - This shall help to insulate the Rila to ensure its thermal constraints of -20℃ to +40℃